# Algae Box - Complete Hardware Shopping List

## 🛒 Complete Bill of Materials (BOM)

Last Updated: January 7, 2026

## 1. Main Controller

### Raspberry Pi 4 Model B (4GB) - $55

- Why: Sufficient processing power, GPIO pins, runs Python

- Alternative: Raspberry Pi Zero 2 W ($15) - cheaper, lower power, still capable

- Supplier:Adafruit, Amazon

- Includes: Quad-core processor, WiFi, Bluetooth, 40 GPIO pins

### MicroSD Card (32GB Class 10) - $10

- Model: SanDisk Ultra 32GB

- Why: OS storage and data logging

- Supplier: Amazon, Best Buy

### Power Supply (5V 3A USB-C) - $10

- Model: Official Raspberry Pi Power Supply

- Why: Stable 5V 3A for Pi + sensors

- Supplier: Adafruit, Amazon

## 2. Sensors

### A. Turbidity Sensor - $12

Option 1: DFRobot SEN0189 (Recommended)

- Model: SEN0189 Gravity Analog Turbidity Sensor

- Range: 0-1000 NTU

- Output: 0-4.5V analog

- Why: Affordable, well-documented, Arduino/Pi compatible

- Supplier:DFRobot, Amazon

- Datasheet:Link

Option 2: TSW-10 Turbidity Sensor - $18

- Higher accuracy, industrial grade

- Supplier: AliExpress, eBay

### B. pH Sensor - $25

Model: DFRobot SEN0161-V2 pH Sensor Kit

- Range: pH 0-14

- Accuracy: ±0.1 pH @ 25°C

- Output: Analog voltage

- Includes: pH probe, BNC connector, signal conditioning board

- Why: Waterproof probe, pre-calibrated, easy Arduino/Pi interface

- Supplier:DFRobot, Amazon

- Note: Requires periodic calibration with buffer solutions

pH Calibration Buffer Set (pH 4.0, 7.0, 10.0) - $12

- Essential for accurate readings

- Supplier: Amazon, scientific supply stores

### C. Temperature Sensor - $8

Model: DS18B20 Waterproof Digital Temperature Sensor

- Range: -55°C to +125°C

- Accuracy: ±0.5°C

- Interface: 1-Wire (single data pin)

- Cable: 1m waterproof stainless steel probe

- Why: Waterproof, accurate, widely supported, simple wiring

- Supplier:Adafruit, Amazon

4.7kΩ Pull-up Resistor (for DS18B20) - $0.10

- Required for 1-Wire protocol

- Included in resistor kit below

## 3. Analog-to-Digital Converter (ADC)

### MCP3008 8-Channel 10-Bit ADC - $4

Model: Microchip MCP3008

- Why: Raspberry Pi doesn't have analog inputs; this converts analog sensor signals to digital

- Channels: 8 (turbidity + pH + 6 spare)

- Resolution: 10-bit (0-1023 values)

- Interface: SPI

- Supplier:Adafruit, Amazon

Alternative: ADS1115 16-Bit ADC - $10

- Higher resolution (16-bit vs 10-bit)

- I2C interface

- Better for precise pH readings

## 4. Actuators & Collection System

### A. Solenoid Valve (Flow Control) - $15

Model: 12V DC 1/2" Electric Solenoid Valve

- Size: 1/2 inch (DN15) or match your tank plumbing

- Voltage: 12V DC

- Type: Normally Closed (NC) - closes when power off (safe default)

- Material: Plastic or brass body

- Why: Stops water flow for settling phase

- Supplier:Amazon, AliExpress

- Options:

  - 1/4" for smaller tanks

  - 3/4" for larger systems

### B. Servo Motor (Shovel Mechanism) - $12

Model: MG996R High Torque Metal Gear Servo

- Torque: 11 kg·cm @ 6V

- Rotation: 180° (0-180°)

- Voltage: 4.8-7.2V

- Why: Strong enough to operate shovel/scraper mechanism

- Supplier:Amazon, hobby stores

Alternative: Linear Actuator - $25-40

- If you need push/pull motion instead of rotation

- 12V DC, 50-100mm stroke length

### C. Relay Module (2-Channel) - $6

Model: 2-Channel 5V Relay Module

- Channels: 2 (valve + shovel power)

- Input: 5V control from GPIO

- Output: Switches 12V DC for valve/servo

- Isolation: Optocoupler isolated

- Why: Safely control high-power devices from Pi GPIO

- Supplier:Amazon, electronics stores

## 5. Power Supply & Distribution

### 12V DC Power Supply (2A minimum) - $12

- Model: 12V 2A wall adapter

- Why: Powers solenoid valve and servo

- Supplier: Amazon

- Note: Get one with barrel jack connector

### DC-DC Buck Converter (12V to 5V) - $5(Optional)

- If you want to power Pi from same 12V supply

- Step-down converter 12V → 5V 3A

- Supplier: Amazon, Adafruit

### Breadboard Power Supply Module - $3

- Converts 12V/9V to 5V and 3.3V rails

- Supplier: Amazon

## 6. Wiring & Connectors

### Jumper Wires (Male-to-Male, Male-to-Female, Female-to-Female) - $8

- Quantity: 120 pieces assorted

- Length: 10-30cm

- Why: Connect sensors to Pi GPIO and breadboard

- Supplier:Amazon

### Breadboard (830 tie-points) - $5

- Why: Prototype circuits before soldering

- Supplier: Amazon, Adafruit

### Dupont Connector Kit - $10

- Male/female pins and housings

- For making custom cable lengths

- Crimping tool included

### Screw Terminal Blocks - $6

- Type: 2.54mm pitch, 2-3 position

- Quantity: 10-20 pieces

- Why: Secure sensor wire connections

- Supplier: Amazon

### Heat Shrink Tubing Set - $8

- Various sizes for protecting connections

- Supplier: Amazon

### Waterproof Cable Glands - $8

- Size: PG7 or PG9

- Quantity: 5-10 pieces

- Why: Seal sensor cables entering tank

- Supplier: Amazon

## 7. Electronic Components

### Resistor Kit (1/4W, 1Ω to 1MΩ) - $10

- Quantity: 600 pieces, 30 values

- Includes: 4.7kΩ for DS18B20, others for circuits

- Supplier: Amazon

### Capacitor Kit (Ceramic & Electrolytic) - $12

- For noise filtering and power stability

- Supplier: Amazon

### Diodes (1N4007) - $3

- Quantity: 20 pieces

- Why: Flyback protection for relay/motor

- Supplier: Amazon

## 8. Mechanical Parts (Shovel/Scraper Mechanism)

### Acrylic Sheet (3mm thick) - $10

- Size: 300x200mm

- Why: Build custom shovel/scraper blade

- Supplier: Hardware store, Amazon

### Stainless Steel Sheet (0.5mm) - $8(Alternative)

- More durable for scraper

- Supplier: Hardware store

### M3 Bolts, Nuts, Washers Set - $10

- Various lengths (6-30mm)

- Mounting hardware

- Supplier: Amazon, hardware store

### Hinges (Small, Stainless Steel) - $5

- For shovel opening mechanism

- Supplier: Hardware store

### Silicone Tubing (Food Grade) - $8

- Size: 1/2 inch ID

- Length: 2-3 meters

- Why: Connect valve to tank drainage

- Supplier: Amazon, brewing supply stores

## 9. Enclosure & Protection

### Waterproof Project Box (IP65) - $15

- Size: 200x120x75mm

- Why: Protect Raspberry Pi and electronics from humidity

- Features: Clear lid, cable entry ports

- Supplier:Amazon

### Silica Gel Desiccant Packets - $6

- Keep moisture out of electronics box

- Supplier: Amazon

## 10. Optional Upgrades

### Camera Module (Algae Identification) - $25

- Model: Raspberry Pi Camera Module V2 (8MP)

- Why: For your algae identification program

- Supplier:Adafruit, Amazon

### USB Microscope - $30-50

- Better for detailed algae imaging

- 1000x magnification

- Supplier: Amazon

### Real-Time Clock (RTC) Module - $6

- Model: DS3231 RTC

- Why: Keep accurate time when Pi offline

- Supplier: Amazon

### UPS HAT for Raspberry Pi - $25

- Model: Geekworm X728 UPS

- Why: Backup power, safe shutdown during power loss

- Supplier: Amazon

### WiFi Range Extender - $20(if needed)

- If tank is far from router

- For remote monitoring

## 11. Tools (If you don't have)

### Soldering Iron Kit - $25

- For permanent connections

- Includes solder, stand, tips

- Supplier: Amazon

### Multimeter - $15

- Essential for troubleshooting

- Measure voltage, continuity, resistance

- Supplier: Amazon, Harbor Freight

### Wire Stripper/Crimper - $12

- For preparing wires

- Supplier: Amazon

### Precision Screwdriver Set - $10

- For Pi and electronics assembly

- Supplier: Amazon

## 📦 Package Recommendations

### Starter Package (Minimum Viable System) - ~$230

✅ Raspberry Pi 4 (4GB) + SD card + power supply
 ✅ DFRobot Turbidity Sensor
 ✅ DS18B20 Temperature Sensor
 ✅ MCP3008 ADC
 ✅ 2-Channel Relay Module
 ✅ 12V Solenoid Valve
 ✅ MG996R Servo Motor
 ✅ 12V Power Supply
 ✅ Jumper wires + breadboard
 ✅ Basic resistor kit
 ✅ Waterproof enclosure

Skip pH sensor initially, add later when calibration solutions available

### Complete System - ~$380

Everything in Starter Package PLUS:
 ✅ DFRobot pH Sensor + calibration buffers
 ✅ ADS1115 (higher precision ADC)
 ✅ Camera module
 ✅ RTC module
 ✅ All mechanical parts
 ✅ Tools (if needed)

### Professional Grade - ~$500

Complete System PLUS:
 ✅ UPS HAT
 ✅ USB Microscope (better imaging)
 ✅ Industrial turbidity sensor
 ✅ Linear actuator (instead of servo)
 ✅ Quality soldering kit

## 🌐 Recommended Suppliers

### Primary (US):

- Adafruit - High quality, excellent documentation

- Amazon - Fast shipping, variety

- DFRobot - Direct from manufacturer

### Budget Options:

- AliExpress - Cheap, but 2-4 week shipping

- eBay - Used/surplus components

### Specialty:

- Digi-Key / Mouser - Electronic components, professional

- McMaster-Carr - Mechanical parts, industrial quality

## 📋 Shopping Links by Category

### All-in-One Kits to Consider:

Raspberry Pi Starter Kit - $100

- Includes Pi, case, power supply, SD card, heatsinks

- Example: CanaKit Raspberry Pi 4 Starter Kit

Sensor Bundle - $80

- Turbidity + pH + Temperature all together

- Search: "Arduino water quality sensor kit"

## ⚡ Wiring Diagram Coming

When you're ready to order, I can provide:

- Complete wiring diagram showing all connections

- Pin assignments for each component

- Assembly instructions step-by-step

- Updated code with your specific sensor models

## 💰 Total Cost Estimate:

| Configuration | Cost | Timeline |
| --- | --- | --- |
| Minimum Viable | $230 | Order now, start with basics |
| Recommended | $380 | Full featured, room to grow |
| Professional | $500 | Production quality |

Note: Prices are approximate USD as of January 2026. Check current prices as components fluctuate.

## 🚀 Recommended Ordering Strategy

### Phase 1 - Order Now:

- Raspberry Pi + accessories

- Turbidity sensor

- Temperature sensor

- MCP3008 ADC

- Relay module

- Solenoid valve

- Basic wiring kit

Start coding and testing while waiting for:

### Phase 2 - Order Later:

- pH sensor (requires calibration knowledge)

- Servo/actuator (after designing shovel mechanism)

- Camera module (for algae ID integration)

- Mechanical parts (custom to your tank)

Want me to create:

- Detailed wiring diagram for specific components?

- Assembly instructions for the shovel mechanism?

- Calibration procedures for each sensor?